# OmniView IQ: Unified Industrial IoT & Enterprise Analytics Platform

## 1. The Unified Platform Thesis

The industrial energy landscape is characterized by a fundamental operational disconnect: facility engineering teams monitor machine health independently of financial consequence, while plant management views energy consumption reactively upon receiving utility bills. Decisions are thus made as delayed reactions to costs that have already been locked in.

Originally bifurcated into two distinct product propositions—one focused on electricity billing pain ("EcoGrid") and the other on comprehensive factory telemetry ("OmniView")—the platform has been definitively consolidated into a singular, unified architecture: OmniView IQ.

OmniView IQ is a ground-up, end-to-end Industrial Internet of Things (IIoT) operating system designed for environments with zero existing digital infrastructure. It utilizes non-invasive sensors buffering to a localized edge gateway, which then transfers data to the cloud for computation.

## 2. Realistic Default Values & Operational Baselines

To execute a successful deployment within a 2-to-3 week window without relying on existing PLCs or SCADA systems, the system models its anomaly detection against realistic default thresholds common in heavy manufacturing (specifically modeled on Single-Stage ISBM PET machinery like the Nissei ASB-70DPH)1.

- Apparent Power & Demand (kVA): Baseline factory connected load is assumed at 75–150 kW per line. Contracted Maximum Demand (MD) is assumed at 500 kVA.

- Mechanical Vibration (ISO 10816-3): For medium-sized Class II rotating machinery (15kW to 300kW) mounted on rigid foundations, continuous RMS velocity up to 2.8 mm/s is "Normal". Between 2.8 mm/s and 7.1 mm/s is "Marginal/Restricted". Anything above 7.1 mm/s is "Critical" and actively causing bearing damage3.

- Pneumatic Pressure: High-pressure multi-stage compressors operate between 25 and 40 bar. A slow decay in pressure while the compressor is loaded indicates invisible pneumatic leaks, which typically waste 20% to 30% of compressor energy6.

- Thermal Ranges: ISBM injection barrel heating zones normally maintain equilibrium at 250°C to 260°C.

## 3. Financial Assumptions (MSEDCL Tariff)

Because facility finance teams often withhold exact billing details during initial pilots, the platform assumes standard Maharashtra State Electricity Distribution Co. Ltd. (MSEDCL) HT-I Industrial Tariffs:

- Demand Charge: ~INR 400 per kVA per month, billed against actual recorded maximum demand or 75% of contracted demand7.

- Energy Charge: ~INR 5.47 to INR 6.85 per kVAh7.

- Time of Day (ToD): 20% surcharge during evening peaks (17:00 - 24:00) and up to 25% rebate during solar hours (09:00 - 17:00)7.

- Penalty Implication: A single uncoordinated machine start-up exceeding the 500 kVA contract limit for a 15-minute rolling window triggers a permanent monthly penalty ranging from INR 15,000 to INR 40,000.

## 4. Hardware Inventory & Edge Architecture

To achieve absolute independence from the facility's legacy systems, OmniView IQ installs its own non-invasive hardware network.

| Sensor / Hardware Component | Make & Model | Purpose & Monitored Parameter | Est. Cost (INR) | Availability |
| --- | --- | --- | --- | --- |
| Edge Gateway / Central Unit | Teltonika RUT956 | Modbus polling, offline flash buffering, MQTT cloud bridge via 4G LTE. | 22,000 – 29,000 | High (Standard Stock)9 |
| Digital Energy Meter | Selec MFM384-C-CE | 3-Phase power, kVA, kW, PF, THD up to 31st harmonic. | 4,000 – 6,000 | Very High12 |
| Current Transformers | Selec SCCT-30/20 | Non-invasive split-core CTs; snaps around live cables without stopping production. | 1,500 – 2,000 / phase | Very High14 |
| Vibration & Temp Node | Banner Q45 or NCD | Wireless MEMS triaxial accelerometer capturing ISO 10816-3 RMS velocity and surface temp. | 25,000 – 35,000 | Medium (1-2 Weeks)16 |
| Pressure Transmitter | WIKA A-10 (40 bar) | Analog 4-20mA sensor tapped into existing gauge ports to track pressure decay. | ~15,000 | High (Pune local)17 |

## 5. The 6-Layer Operating System Architecture

- Layer 0 (Site Configuration Profile): The JSON genome of the factory containing multi-tenant rules. No downstream layer contains hardcoded data.

- Layer 1 (Sense): Protocol-specific adapters pulling telemetry from the standalone sensors listed above via RS-485 and Modbus RTU.

- Layer 2 (Connect): The Teltonika Edge Gateway. It synchronizes clocks via NTP to ensure the 15-minute billing window perfectly aligns with utility meters. It provides the "Offline Buffer"—if 4G drops, data caches locally and chronologically backfills upon reconnection.

- Layer 3 (Model): The analytics and Digital Twin engine. It executes the Cold-Start Baseline: Day 1 uses static nameplate heuristic rules (e.g., >7.1 mm/s vibration), Day 5 uses statistical baseline tracking, and Day 10+ locks in the fully calibrated Machine Learning predictive baseline.

- Layer 4 (Intelligence): The business logic and What-If simulation sandbox. It runs Component D5 (Conflict Arbitration Logic) to decide actions when maintenance and energy goals contradict.

- Layer 5 (Monetize): Financial computations. Translates kWh to Rupees via MSEDCL tariffs and computes Estimated CO2e Avoided for ESG reporting.

- Layer 6 (Deliver & Act): The notification routing engine enforcing Human-in-the-Loop (HITL) hierarchy before execution.

## 6. Alerting Architecture & Stakeholder Routing

OmniView IQ avoids alert fatigue by contextually routing data only to the stakeholder capable of acting on it.

| Target Stakeholder | Primary Data Focus | Example Alert Trigger | Delivery Mechanism |
| --- | --- | --- | --- |
| Plant Floor Operator | Machine safety, OEE | Sudden breakdown; ISO 10816 Zone D threshold breach (>7.1 mm/s). | Direct SMS; Flashing red indicator on floor jumbo display. |
| Maintenance Engineer | Remaining Useful Life (RUL), vibration/thermal drift | ISO 10816 Zone C entry (5.1 mm/s); Motor running current drifting 10% above normal. | Automated CMMS ticket; detailed email with asset history. |
| Facility Manager | 15-min MD forecasts, load scheduling | Rolling kVA forecast reaching 95% of contract limit at minute 11 of the 15-min window. | High-priority mobile app push; Dashboard "What-If" action card. |
| Finance / ESG Officer | Accrued ToD costs, ROI, Carbon | Month-end penalty tracking; Incremental ToD rebates achieved. | Scheduled PDF/Excel automated MIS generation. |

## 7. System Affection & Granular Edge Cases

The system maps cascading value changes dynamically. Below are three escalating edge cases detailing exactly how the system reacts.

### 7.1 Small Case: Network Partition / Sensor Disconnect

- Scenario: A forklift severs the RS-485 cable to the chiller's energy meter.

- System Affection: Layer 2's buffer instantly tags the node as OFFLINE_UNPLAGGED.

- System Action: Layer 3 does not crash the facility forecast; it temporarily interpolates the chiller's expected kVA based on the historical baseline. Layer 6 strictly routes an alert to the IT/Network admin with the exact timestamp of failure, shielding the Plant Manager from irrelevant packet-loss noise.

### 7.2 Mid-Cycle Case: The Unplanned Machine Shutdown

- Scenario: A 150 kW furnace unexpectedly trips offline at 14:15. An operator tries to delete the machine from the dashboard to "clean up" the error view.

- System Affection: Layer 0's HITL governance strictly blocks single-user machine deletion.

- System Action: Layer 3 immediately forks the active 15-minute peak demand calculation into two lines: Baseline_Expected_kVA and Drift_Adjusted_kVA. Layer 6 generates a "Drift Penalty" alert, calculating the exact financial loss in production and unused energy overhead caused by the sudden trip, delivering facts for the post-mortem analysis.

### 7.3 Worst Case: The Conflict Arbitration Stress Test (Component D5)

- Scenario: At 19:30 (MSEDCL Peak ToD Hours), the facility is drawing 480 kVA of its 500 kVA limit. Suddenly, the primary high-pressure compressor's bearing vibration spikes to 14.5 mm/s (Critical Zone D).

- The Conflict: Layer 3 (Maintenance) demands an emergency compressor shutdown to prevent a rotor explosion (Safety). However, shutting it down causes a pressure collapse, forcing three backup pumps to immediately spool to 100% load. This will spike the 15-minute rolling average to 550 kVA, guaranteeing a ₹40,000 utility penalty (Compliance).

- System Action (Component D5): Layer 4 evaluates the Safety > Compliance > Cost hierarchy. Using the What-If isolated sandbox, it tests a resolution.

- Resolution: Instead of a hard trip, the system calculates a 4-minute staggered deceleration of the compressor while identifying an active 50 kW HVAC unit that can be temporarily curtailed. It generates an immediate HITL Action Card: "Shed HVAC load now. Decelerating compressor over 4 minutes to safely transition pressure. Action avoids catastrophic rotor failure AND prevents ₹40,000 MSEDCL penalty."

## 8. The 3-Week Execution Plan

- Week 1 (Cold-Start & Baseline): Install the Teltonika gateway, Selec meters, and split-core CTs with zero production downtime. The system captures actual energy bleed during scheduled idle shifts ("phantom runs") to establish the undeniable financial baseline.

- Week 2 (Algorithmic Calibration): The Layer 3 ML engine matures past Day 10. The rolling 15-minute kVA calculation runs in parallel with the utility meter to capture uncoordinated load overlaps (e.g., heaters and compressors starting together).

- Week 3 (Financial Proof & Delivery): Populate the executive dashboard with real data, verifying the Specific Energy Consumption (SEC) and exact ₹ values of demand penalties avoided. Final handover to plant management.

#### Works cited

- ASB-70DPH ASB-70DPW | Nissei ASB, https://www.nisseiasb.co.jp/en/products/catalogs/pdf/asb70dph-asb70dpw_201206_en.pdf

- ASB-70DPH ASB-70DPW | Nissei ASB, https://www.nisseiasb.co.jp/en/products/catalogs/pdf/asb70dph-asb70dpw_201510_en.pdf

- ISO 10816 Vibration Limits Overview | PDF | Home & Garden | Computers - Scribd, https://www.scribd.com/doc/100941122/93616775-Vibration-Limits-as-Per-Iso-10816-Standards-2

- ISO 10816-3 Vibration Severity Chart | PDF | Technology & Engineering - Scribd, https://www.scribd.com/doc/238323686/velocity-chart-pdf

- Understanding the ISO 10816-3 Vibration Severity Table - DSP Analytic, https://dspanalytic.com/en/vibrations/understanding-the-iso-10816-3-vibration-severity-table/

- Compressed Air Leak Detection – Ultrasonic - Elion Technologies, https://elion.co.in/compressed-air-leak-detection-services/

- Maharashtra Electricity Tariff 2025-26 | PDF - Scribd, https://www.scribd.com/document/869024051/Tariff-Revision-for-FY-2025-26-to-FY-2029-30

- Solar + BESS Payback for Factories in India: 2026 CFO Guide | MGetEnergy, https://www.mgetenergy.com/blogs/solar-bess-cfo-avoided-cost-2026/

- Teltonika RUT955 Cellular Wifi Industrial Router Service Provider from Pune - Prosoftek Corporation, https://www.prosoftektracking.com/industrial-router.html

- Teltonika RUT906 Industrial Router IoT Gateway 4G Service Provider from Pune - BluBoxx Communication Private Limited, https://www.bluboxx.in/teltonika-industrial-router-iot-gateway-vpn-router.html

- Teltonika RUT 956 LTE RS 232/RS 485 Industrial 4G LTE (Cat 4), 3G, 2G - Grabnpay, https://www.grabnpay.in/products/teltonika-rut-956-lte-rs-232-rs-485-industrial-4g-lte-cat-4-3g-2g-cellular-router-with-dual-sim-wlan-gps-openvpn-dyndns

- Selec MFM384-C-CE Multifunction Meter - Parma India, https://parmaindia.com/selec-mfm384-c-ce-8-digits-lcd-display-three-phase-digital-multifunction-meter-rs485-communication-pulse-output-size-96x96mm-panel-mount.html

- Selec Multifunction Meter - Latest Price, Dealers & Retailers in India - IndiaMART, https://dir.indiamart.com/impcat/selec-multifunction-meter.html

- Selec SCCT-30/20-300/5A-VA2.5-CL0.5-CE Split Core Current Transformer - ElectrixZone, https://electrixzone.com/product/selec-scct-30-20-300-5a-va2-5-cl0-5-ce-split-core-current-transformer/

- Selec SCCT-30/20-100/5A-VA1-CL1-CE Current Transformers-Split Core at ₹ 1500/piece | in Mumbai | ID: 2853125860512 - IndiaMART, https://www.indiamart.com/proddetail/selec-scct-30-20-100-5a-va1-cl1-ce-current-transformers-split-core-2853125860512.html

- DX80N9Q45THA Banner Engineering | Mouser India, https://www.mouser.in/ProductDetail/Banner-Engineering/DX80N9Q45THA?qs=vvQtp7zwQdMX0hBrvxCQ6A%3D%3D

- Wika Pressure Transmitter in New Delhi - IndiaMART, https://dir.indiamart.com/new-delhi/wika-pressure-transmitter.html